intelligent image sensor

# iIS-010 series

TK6234A5	iIS-010

TK6238A7	iIS-011

TK6240A5	iIS-012

# Operation Manual
(Application)

Rev. Draft
July 29 2004

Thank you for purchasing our iIS-010 series intelligent image sensor (iIS). Before using your iIS, please read this user's manual 
to become familiar with the correct usage procedures. 
After reading, keep this manual in a safe place for future reference.

This operation manual is manufactured from recycled paper.

## TABLE OF CONTENTS

TABLE OF CONTENTS	1

Creating An Inspection Solution	3

Configuring Your Application	3

Exposure	7

Things to know about Processing Time	8

Gauge	9

Origin Properties - Optional	11

Locator Properties - Optional	12

Using Edge Sensitivity	16

Using Perpend (icular) Line-Line	17

Using Line-Line	18

Using Point-Point	20

Using Point-Line	21

Using Angle	22

Using Corner	24

Using Circle	25

Using Circle-Circle	27

Using Circle-Line	28

Using Point-Circle	29

Using Concentric Circles	30

Using Count	31

Using Edge Count	32

Using Intensity	33

Label	35

Set up the Inspection	35

Set up the Inspection Area	36

Set up Barcode Readings	38

Set up Inspection Settings	39

Defect Tolerance in Inspection Settings versus Learn Images	40

Setup Model Learning	41

Readjust the Acceptable Contrast Tolerance	42

Setup Decision Logic	43

Set up Outputs	44

Set Up Rejector Timing	45

Rejector Delay	46

Rejector Setup Procedure	47

Set Up Reporting	49

Create Condition	50

Select or Define New Strings	51

Select or Define New Destinations	52

Ready to Start Processing	53

Monitor Inspection Results	54

Review History	56

Save and/or Export your Solution	57

Resetting the control unit	58

Close the Application Window	58

Monitoring Inspections from the Web	59

Viewing Data Output	60

History Log	60

HyperTerminal	60

## Creating An Inspection Solution

ｚThe best way to learn iIS is to create a “Solution”. A Solution is a configured application that performs an inspection of a specific part. The Solution is stored on the control unit and automatically runs when the control unit is powered on, independent of any network connection. The following presents a guide through the process of creating and saving a Solution on your iIS. Not all of the steps covered are necessary for every application, some are optional, but we recommend going through them once to familiarize yourself with the product features and capabilities. The INFO button at the bottom of each panel (menu or monitor screen) gives help information on the panel options. You can also navigate through Help using hyperlinks, the Table of Contents, Index, or Search.

## Configuring Your Application

1. 	Access the control unit

If you are not already at the iIS home page, open Internet Explorer and reenter the IP Address of your iIS control unit.

2. 	Launch the Application

Click on “Gauge” or “Label”.

The iIS Home Page (Gauge)

The iIS Home Page (Label)

NOTE	The first time you open this Gauge/Label page, you will get a warning message about certificates and signatures. Click OK. The control unit is writing its OCX and support files into your System directory. A few seconds after the hourglass disappears, the iIS Application Window will open.

iIS is easily configured through a series of setup and control screens that are intuitively linked. The top-level or home screen (following image) provides Solution management (start, load, setup/modify and save), as well as monitoring (reviewing run-time results). Each screen provides active and inactive (grayed) buttons that are connected in a logical progression to help guide you through the correct steps. Inactive buttons, such as “monitor” will only become active when their dependent steps have been completed, in this case “setup”.

Each screen has two panels. The left panel is for Control and Navigation, with the Action Buttons. The right panel is the Work Area, and displays the camera image. These two panels are always present in the application window, but most of the following figures will show only one of the two panels.

The bottom of the home Control and Navigation panel has Info and Close buttons. The Close button will close out the Application window, but leave the control unit running and inspecting. All other Control and Navigation panels have Info and OK buttons. The OK button exits a panel and goes back up one level.

Control &

Navigation

Image &

Work Area

The Gauge Application Window

Control &

Navigation

Image &

Work Area

The Label Application Window

3.	Start a Solution

a.	Click on the “Select Solution” button.

b. Since you are defining a new Solution, click the “Start New Solution” button. If you had an existing Solution configured, this action would overwrite it. The iIS control unit will initialize, exit this screen, and go to the Setup panel.

Current Solution ID number

Start creating a new Solution (application) and overwrite any existing Solution in the iIS control unit memory. Initializes the control unit and exits this screen.

Load the last Solution saved at ID 00 in control unit memory.

Assign a number if you have multiple Solutions. Up to 16 Solutions can be saved on the control unit. See note below.

Initialize the iIS and exit this panel.

Load a previously saved Solution, from this PC or the network, into control unit memory.

Progress Bar

Select Solution Panel

NOTE	Up to 16 Solutions can be saved on the control unit. A Solution imported as ID = 00 becomes the current (running) Solution and is stored in control unit memory as Solution 00. Solutions imported and assigned IDs 01 through 15 are stored in control unit memory, but do not become the current running Solution. To run Solutions 01 through 15, you must supply a 4-bit binary ID code and a “change solution” pulse at the I/O connector. An example of the switch is shown on page 83. If you do not have a way to supply the 4-bit code, each Solution should be Saved and Exported to the PC or network. You can import any Solution as ID = 00 and run it.

4.	Set up a Solution

In the Setup screen, only the Sensor button is active. This step must be completed before any of the other screens can be accessed. If you had loaded a previously saved Solution, all of the buttons would be active.

a.	Click on the “Sensor” button to access the Setup Sensor panel.

Set up sensor (camera), trigger, and image parameters.

Not active for “Start New Solution” because the Sensor must be set up first.

Exit this panel and go back one level.

Setup Panel

5.	Set up the Sensor

Here you can to set up the sensor input (trigger) and the sensor image settings. If you are using a conveyer or other moving parts, set up the trigger delay, exposure and brightness with moving parts.

Select what causes the iIS to snap an image:

Internal Timer-an internal signal from the control unit clock. The slider defines time between images.

Inspection Trigger-an external signal from a photosensor or a relay.

Click the box to enable the slider, to set a delay between when the object is under the trigger sensor and when the object is under the camera.

Define the duration of a pulse (pulse width) and offset (delay) after a trigger input, for controlling light illuminating your object.

If you have a monitor connected directly to the back of the iIS, select how often this display is updated:

Continuous for focusing the camera, None for fastest operation.

You can adjust the Exposure with the slidebar.

You can adjust the sensor settings to get the best image. Invert Contrast reverses black and white, or makes the image a negative.

(Brightness supports very slight variable)

Calibration is optional (Gauge application). If you do run Calibrate, the iIS reports measurements in real units.

Click OK when you are finished, to exit this panel.

Sensor Setup Panel

## Exposure

The Exposure slider range is 1 to 1024. Multiply this by 60 us (microseconds) for the approximate time. Refer to For fast moving parts, you must have a very low exposure, and a very bright light source.

## Things to know about Processing Time

If the internal timer setting, or the time between parts on the line, is less than the total processing time, images may be dropped (resulting in missed inspections)

	The total processing time is: Inspect Time plus 40 ms (approximately) plus client or network overhead. The inspect time is displayed on the Monitor/Run panel.

	Complex objects take more processing time, because the iIS must find all the edges in the image. Light reflecting off shiny metal objects appears as more edges. In some cases, the number of “false” edges can exceed the amount of memory in the control unit.

	When the location or angle of the object or the label position (in the camera view) changes significantly from part to part, the iIS must search for and find the object in the image area. Increased search time causes longer Inspect Time.

	More measurements take more time.

	Increasing the number of sample lines (in the Measurement Properties box) takes more time.

	Updating the Local Display attached to the iIS control unit can add to the overhead. We recommend disabling image updates in run mode.

	Updating the Client (your PC) display and network traffic can add to the client of network overhead. We recommend disabling image updates in run mode.

	Recording data and images to History can add to the overhead. We recommend using history during development of the application and disabling it in run mode if possible. Note that viewing history during run mode could result in missed inspections if the timing between parts is tight. For this reason, history is always disabled when the application is monitored from the browser.

On the Gauge application, if you are using the internal timer, increase the Sensor Trigger slider to 100 ms for a simple part with few measurements, or increase to 300 ms for a complex part or if you are making several measurements.

On the Label application, if you are using the internal timer, increase the Sensor Trigger slider to 150 ms for a simple label with little print, or increase to 250 ms for a complex label.

Once you have set up the sensor and locator, defined your measurements, and completed the Solution setup; run your Solution and notice the “Inspect Time” on the Run screen. The internal timer must be larger than the total processing time. First save your Solution with the internal timer set to 150% of the processing time, then you can go back and try adjusting the time to a smaller interval. Notice the Parts Skipped counter on the Run panel.

At 5 parts per second or higher, you may not see every part on the client (programming) PC display area. You must check the Skipped Parts count on the Run panel to see if every part is inspected. Or, examine History Recall.

## Gauge

1.	Running Calibrate - Optional

A calibration sheet (Two “checkerboard” patterns) is attached to this product.

a.	Place a black and white checkerboard in front of the camera, at the same distance where your inspection parts will be.

b.	Type the size of a single square in the field next to Cell Size. Do not type units, just a number. For example, if one square is 10 mm by 10 mm, type the number 10.

c.	Click the Calibrate button. The iIS will compute the correction information, then draw red boxes around the squares in the checkerboard. An X,Y axis origin appears at the top-left corner of the first complete (whole) box in the checkerboard image.

d.	Click OK to exit this screen and go back one level to the Sensor Setup.

Cursor Location and pixel intensity

Calibration Panel

Complete the Sensor Setup and click OK. With Sensor Setup completed, a check appears next to the Sensor button, and the “Gauge” button is now active.

2.	Set up Measurements

a.	Click on Gauge.

b.	Make sure a good (or perfect) part or assembly is in front of the camera, in the location where your inspection parts will appear, or a series of good parts moving past the camera.

c.	Click on “Take a Picture” and look at the still image. You will use this image to set up your measurements and tolerances. If you change the lighting or exposure you will need to take a new picture.

First, take a picture of the part. You want a “still” or “frozen” picture of a good part, or even a “perfect” part, to set up your measurements. Your production will be measured against this part.

Exit the measurement setup panel and go back up one level.

Next you tell the iIS about how much the position of your production parts may vary from the example you you train on.

Next you set up your measurements and the tolerances.

You can change colors of the measurement tools and graphics to suit your preference.

You can define a path to save an image on your PC.

Measurements Setup

3.	Set Up Locator

a.	Click on Locator.
In the Locator setup, you tell the iIS how much the position of inspected parts will vary. The more parts vary in position, the more time it takes to locate the part before performing the inspection. The optimal situation is when the parts are mechanically placed or fixtured, and appear in exactly the same position without any change in rotation, twist, height, etc. The defaults are set for no variation in part position.

b. To allow variation in position, click the radio button beside “Confined to a region”. Two boxes appear in the image display. Move and resize the smaller (inside) box to define a unique feature (a registration mark, or “fiducial”) on the part, and resize the larger (outside) box to define the area to search for that feature.

c.	To allow a rotation, click the radio button beside “ a few degrees”. A second pair of boxes appear. Use them to define a second unique feature, and a search area for that feature.

d.	Complete the Locator setup, and click OK to return to the Measurements panel.

Specify if the inspection parts vary in their position, up and down or left and right.

Specify if the inspection parts vary in orientation or rotation.

Tip Window - gives hints on drawing locator.

Exit this panel and go back one level.

Locator Panel

### Origin Properties - Optional

When you hover your cursor over the axis origin, the axis turns red and a message appears telling you to “right-click to edit”. You can drag the origin to another location when it is red. Right-click on the origin. A Origin Properties edit box opens. You can change the location of the origin. The default is positive rotation in the Clockwise direction. changing position or changing sign of the X,Y axis may change the direction of rotation.

Origin Properties

Relative to current origin	move the origin from the current location by the numbers entered in the X,Y fields. Numbers are in calibrated units, or in pixels if Calibration was not performed.

Absolute image coordinates	move the origin to the absolute pixel coordinate using the numbers in the X,Y fields.

Upper left	move the origin to the top-left corner of the image area. Return to absolute pixel coordinates. Positive rotation is Clockwise.

Upper right	move origin to the top-right corner of the image area. Return to absolute pixel coordinates. Positive rotation is Counter-Clockwise.

Lower right	move origin to the bottom-right corner of the image area. Return to absolute pixel coordinates. Positive rotation is Clockwise.

Lower left	move origin to the bottom-left corner of the image area. Return to absolute pixel coordinates. Positive rotation is Counter-Clockwise.

### Locator Properties - Optional

When you hover your cursor inside one of the Search Area boxes, a message appears telling you to “right-click to edit”. Right-click in the search box, and a Locator Properties edit box opens. You can enable parameters and set tolerances for Pass/Recycle/Fail conditions, or for content of output messages. Parameters or measurements that are not enabled cannot be used for message output.

Locator Properties

Confidence	a “quality match” or “score” (in percent) indicating how much the found feature (on the inspected part) matches the feature on the original.

Position	variation in either horizontal or vertical position. Units are in calibrated units, or in pixels if Calibration was not performed or axis of origin was reset to a corner of the image area.

X	variation in horizontal position only. Units are in calibrated units, or in pixels if Calibration was not performed or axis of origin was reset to a corner of the image area.

Y	variation in vertical position only. Units are in calibrated units, or in pixels if Calibration was not performed or axis of origin was reset to a corner of the image area.

Rotation	variation in rotation, in degrees. Direction is positive from X axis to Y axis. This direction is dependent on position of the origin of axis.

NOTE	If you get frequent warnings, recycles or failures by the part locator, you may need to lower the Confidence setting, or improve the image quality.

4.	Set up Tools

a.	Click on “Inspect Tools”. The tools set up the actual measurements. There is a second panel of More Tools. You can set up as many measurements as you need, with multiple tools. Valid edge points are highlighted as you move the mouse over the image. The valid edge type changes with the tool in use.

b.	Click OK when you are finished defining your measurements. The following pages describe how the tools are used.

Perpend Line-Line	-	measure the perpendicular distance between two edges.

Line-Line	-	measure the distance between two edges.

Point-Point	-	measure the distance between two points, circle centers, a corner, or a midpoint.

Point-Line	-	measure the distance between one edge and a corner, a circle center, or a midpoint.

Angle	-	measure the angle between two edges.

Corner	-	define a corner point, or measure the angle between two edges.

Circle	-	define a point, or measure the diameter or circularity of a circle.

Circle-Circle	-	measure the distance between 2 circle centers.

Circle-Line	-	measure the distance from a circle center to an edge.

Point-Circle	-	measure the distance between a circle and a corner point or midpoint.

Concentric Circles	-	measure the distance from each circle center, to the average center.

Eraser	-	delete a measurement.

More Tools	-	another page of tools.

Tip Window	-	Gives hints on drawing measurements.

Tools Setup Panel

NOTE	Perpend Line-Line or Line-Line can also be used to measure from the edge of the video window.

NOTE	Edge Sensitivity is selectable by right-clicking on any highlighted edge. For example, if you have a low contrast image and are having difficulty locating edges, increasing the edge sensitivity may help.

Count	-	count the number of objects in a ROI. **

Edge Count	-	count the edges crossed, along a line or the outline of the ROI. **

Intensity	-	measure pixel intensities in a ROI and calculate statistics. **

Eraser	-	delete a measurement.

ROI	-	select the shape of your "Region-Of-Interest" to draw, for these tools.

Tip Window	-	Gives hints on drawing measurements.

** Measurement is sensitive to change in light conditions.

More Tools Setup Panel

### Using Edge Sensitivity

a.	Move your cursor into the image area. Hover on any edge in the image and a message will appear telling you to “right-click to edit.”

b.	Right-click on an edge. The Edge Sensitivity edit box opens.

c.	Use the slider and arrow keys to adjust the edge sensitivity from 0% to 100%.

d.	Click on the “check” button to accept changes and close the Line Properties box. Click on the red X button to close without changes.

	There is a delay in the response time, because all edges in the image are updated as you change the sensitivity. Allow time for the updates to occur. Do Not continue tapping the arrow keys or moving the slider. The response time continues to increase (get slower) over a setting of 95%.

	Do not attempt to adjust the sensitivity if the Local Display Update Rate is “Every 100” on the Setup Sensor panel. The additional delay caused by two active displays will make adjusting sensitivity very difficult. Change the Local Display Update Rate to “None”.

	Lighting, lens, focus, and exposure all affect the edge sensitivity and the sharpness and number of edges in the image.

	There is a limit to the amount of information (graphics and highlighted edges) the control unit can send to the client (PC). Setting sensitivity to 95% or greater may cause many edges to be drawn in only the top half of the image, and no edges in the lower half. If this occurs, you must set the sensitivity to a lower number. The application will not identify edges in the lower part of the image if the limit is reached, even when the Edge Sensitivity edit box is closed.

	Generally, few or no edges will be displayed for a setting of 50% or lower.

	If you cannot find any edges in the image, hover over the edge of the video window and right-click to get the Edge Sensitivity edit box. Move the slider to anywhere between 70% and 90%, release, and wait for the display to update.

Edge Sensitivity

### Using Perpend (icular) Line-Line

Perpendicular Line-Line measures the shortest (perpendicular) distance between two edges.

Highlighted Edge

2

1

2

2

2

a.	Click on the “Perpend Line-Line” button. Move the mouse over the image in the Work Area. You should see sections of the edges in the image highlighted as you move across them. These are “edge candidates” that you can use with Perpend Line-Line. Valid edges can be very small curves, points, small or large lines, or the edge of the image area.

Gauge Line

Your line

and iGauge draws a “ruler” line between

measures

shortest

distance

b.	Find two edges that match your measurement. Click on one edge, and “rubber” line appears, and follows your cursor movement. Click on the other edge, and iIS draws a “ruler” line between the two edges that are closest to your click points, and usually draws a Right Triangle to find the shortest distance between the two edges. This is your measurement. The line is labeled (L1, L2, etc.).

c.	When the Perpend Line-Line tools is selected, you can drag the endpoints if they are not exactly where you wanted. The small boxes that appear around the endpoints define the search area for locating the edge at each endpoint. You can drag the solid square at one corner of the box, to resize the box and increase the search area; but searching more area increases the processing time.

boxes

boxes

drag here to resize the

search area

d.	When you hover over a ruler line, the line turns red, the measured length is displayed on the image, and a message appears telling you to “right-click to edit.”

e.	Right-click on the line. A Ruler Properties edit box opens. In this box you can: rename the measurement, set the tolerances for recycle and reject, change the “accuracy” (number of samples to find the edge at your endpoints). You can also select Display Midpoint, which enables the midpoint as a point for another measurement.

Ruler Properties

f.	You can click on the ruler icon in the Ruler Properties edit box, to set units on the measurement.

g.	Click on the “check” button to accept changes and close the Ruler Properties box. Click on the red X button to close without changes.

### Using Line-Line

Line-Line measures the distance between two edges.

Highlighted Edge

2

1

2

2

2

a.	Click on the “Line-Line” button. As you move the mouse over the image in the Work Area, you should see small sections of the edges in the image highlighted. These are “edge candidates” that you can use with Line-Line. Valid edges can be very small curves, points, small or large lines, another line, or the edge of the image area.

b.	Find two edges that match your measurement. Click on one, and a “rubber” line appears and follows your cursor movement. Click on the other edge, and iIS draws a “ruler” line between the edges that are closest to your two points. The line is labeled (L1, L2, etc.).

c.	When the Line-Line tool is selected, you can drag the endpoints if they are not exactly where you wanted. The small boxes that appear around the endpoints define the search area for locating the edge at each endpoint. You drag the solid box at one corner of the box, to resize the box and increase the search area; but searching more area increases the processing time.

d.	When you hover over a ruler line, the line turns red, the measured length is displayed on the image, and a message appears telling you to “right-click to edit.”

e.	Right-click on the line. A Ruler Properties edit box opens. In this box you can: change the name of this measurement, set the tolerances for recycle and reject, change the “accuracy”(number of sampling points to find the edge at the ruler endpoints). You can also select Display Midpoint, which enables the midpoint as a point for another measurement.

Ruler Properties

f.	You can click on the ruler icon in the Ruler Properties edit box, to set units on the measurement.

g.	Click on the “check” button to accept changes and close the Ruler Properties box. Click on the red X button to close without changes.

### Using Point-Point

Point-Point measures the distance between two points: circle centers, midpoints, or angle intersections.

a.	Click on the “Point-Point” button. Move the mouse over the image in the Work Area. You should see the points of angles, or the centers of circles, highlighted as you move across them. These are “point candidates” that you can use with Point-Point. Valid points can be Circle centers, corner points or the points where Angle measures meet, or the midpoint of a line measure. (You must define Circles, corners or angles before defining the point-point measurement.).

b.	Find two points that match your measurement. Click on one point, and a “rubber” line appears and follows your cursor movement. Click on the other point, and iIS draws a “ruler” line between these two points. The line is labeled (L1, L2, etc.).

c.	When you hover over the measurement, the line turns red, the measured length is displayed on the image, and a message appears telling you to “right-click to edit.”

d.	Right-click on the line. A measurement and Ruler Properties edit box opens. In this box you can: change the name of this measurement, set the tolerances for recycle and reject, change the “accuracy” (number of sampling points along the line).

Ruler Properties

e.	You can click on the ruler icon in the Ruler Properties edit box, to set units on the measurement. You can also select Display Midpoint, which enables the midpoint as a point for another measurement.

f. Click on the “check” button to accept changes and close the Ruler Properties box. Click on the red X button to close without changes.

### Using Point-Line

Point-Line measures the distance between a point and an edge.

a. Click on the “Point-Line” button. Move the mouse over the image in the Work Area. You should see the points of angles, or the centers of circles highlighted as you move across them. These are “point candidates” that you can use with Point-Point. Valid points can be Circle centers, ruler midpoints, or the points where Angle measures meet.

b. Find a point that match your measurement. Click on the point, then click on the edge that matches your measurement. iIS draws a line between these two points. The line is labeled (L1, L2, etc.).

1

2

Highlighted Edge

c. You can drag the endpoint on the edge if it is not exactly where you wanted. The small box that appears around the endpoint defines the search or sample area for locating the edge at the endpoint. You can drag the solid square at one corner of the box, to resize the box and increase the search area; but searching more area increases the processing time.

d.	When you hover over the line, the line turns red, the measured angle is displayed, and a message appears telling you to “right-click to edit.”

e. Right-click on the line. A measurement and Ruler Properties edit box opens. In this box you can: change the name of this measurement, set the tolerances for recycle and reject, change the “accuracy” (number of sampling points to find the edge). You can also select Display Midpoint, which enables the midpoint as a point for another measurement.

Ruler Properties

f. You can click on the ruler icon in the Ruler Properties edit box, to set units on the measurement.

g.	Click on the “check” button to accept changes and close the Ruler Properties box. Click on the red X button to close without changes.

### Using Angle

Angle measures the angle between two edges.

Highlighted Edge

a.	Click on the “Angle” button. Move the mouse over the image in the Work Area. You should see sections of the edges in the image highlighted as you move across them. These are “edge candidates” that you can use with Angle. Valid edges are straight lines only.

b.	Find two edges that match your measurement. You can draw a line from one edge to any other. iIS will draw a triangle by extending these two edges until they meet, and a curve for the angle between them. It is OK if the extending lines go off the visible area. The curve or arc between the two edges is labeled (A1, A2, etc. for angle).

c.	When the angle tool is selected, you can drag the endpoints if they are not exactly where you wanted. To increase accuracy of the angle measurement, move the endpoints so they are not close together. The small boxes that appear around the endpoints define the search area for locating the edge at each endpoint. You can drag the solid square at one corner of the box, to resize the box and increase the search area; but searching more area increases the processing time. You can drag the curve drawn to represent the angle, to make it look better in the image work area. The endpoints do not change when you drag the curve.

d.	When you hover over the line, the line turns red, the measured angle is displayed, and a message appears telling you to “right-click to edit.”

e.	Right-click on the line. An Angle Properties edit box opens. In this box you can: rename the measurement, set the tolerances for recycle and reject, change the “accuracy” (number of sampling points to find the edges at the endpoints).

Highlighted Edge

1

2

drag here to

resize the

search area

Angle Properties

f.	Click on the “check” button to accept changes and close the Angle Properties edit box. Click on the red X button to close without changes.

### Using Corner

Highlighted Edge

Corner defines a point, or measures the angle between two edges. The default is no measurement enabled.

a.	Click on the “Corner” button. Move the mouse over the image in the Work Area. You should see sections of the edges in the image highlighted as you move across them. These are “edge candidates” that you can use with Corner. Valid edges are straight lines only.

2

1

b.	Find two edges that touch, that match your measurement. You can draw a line from one edge to any other. iIS draws a triangle by using these two edges, and a curve for the angle between them. The curve or arc between the two edges is labeled (A1, A2, etc. for angle).

c.	When the corner tool is selected, you can drag the endpoints if they are not exactly where you wanted. To increase accuracy of the angle measurement, move the endpoints so they are not close together. The small boxes that appear around the endpoints define the search area for locating the edge at each endpoint. You can drag the solid square at one corner of the box, to resize the box and increase the search area; but searching more area increases the processing time. You can drag the curve drawn to represent the angle, to make it look better in the image work area. The end-points do not change when you drag the curve.

drag here to

resize the

search area

d.	When you hover over the line, the line turns red, the measured angle is displayed on the image, and a message appears telling you to “right-click to edit.”

e.	Right-click on the line. An Angle Properties edit box opens. In this box you can: rename the measurement, set the tolerances for recycle and reject, change the “accuracy” (number of sampling points to find the edges at the endpoints). Note that Corner defaults to no measurement enabled.

Angle Properties

f.	Click on the “check” button to accept changes and close the Angle Properties edit box. Click on the red X button to close without changes.

### Using Circle

Highlighted Edge

Circle defines a point, or measures the diameter or circularity (roundness) of a circle. The default is no measurement enabled.

a.	Click on the “Circle” button. Move the mouse over the image in the Work Area. You should see sections of the edges in the image highlighted as you move across them. These are “edge candidates” that you can use with Corner. Valid edges are arcs, curves and circles only.

Circle and

Center point

b.	Find a curve or circle, that matches your measurement, and click on it. If it is close enough to forming a circle, iIS will draw a circle around the curve, and a center point, and measure the diameter of the circle. The circle is labeled (C1, C2, etc. for circle).

c.	When the circle tool is selected, a pair of blue circles appear, one inside and one outside the measured circle, when you move your cursor across the circle. These blue circles define the search area for locating the edges of the circle. You can drag these circles to resize them and increase the search area; but searching more area increases the processing time.

d.	When you hover over the circle, the circle turns red, the diameter are displayed on the image, and a message appears telling you to “right-click to edit.” Several radial lines appear at even spacing around the circle. These are “sample lines” for finding the edges of the circle.

drag here to

resize the

search area

e.	Right-click on the highlighted circle. A Circle Properties edit box opens. In this box you can: rename the measurement, set the tolerances for recycle and reject, change the “accuracy”(number of sampling points around the circle). Use the tabs and “Enable” boxes to select Diameter or Circularity. Circularity is the ratio of the smallest diameter to the largest diameter, sampled around the circle. Note that Circle defaults to no measurement enabled.

Circle Properties

Diameter	measure the diameter of the circle.

Circularity	measure the ratio of the smallest diameter to the largest diameter, sampled around the circle.

f.	You can click on the ruler icon in the Circle Properties edit box to set units on the measurement.

g.	Click on the “check” button to accept changes and close the Circle Properties edit box. Click on the red X button to close without changes.

### Using Circle-Circle

Circle-Circle measures the distance between two circle center points. You need to have already defined circles using the Circle tool.

a.	Click on the “Circle-Circle” button. Move the mouse over the image in the Work Area. You should see the centers of circles highlighted as you move across them. These are “point candidates” that you can use with Circle-Circle. Valid points are Circle centers.

b.	Find two circles that match your measurement. Click on one center point, and a “rubber” line appears that follows your cursor movement. Click on the other center point, and iIS draws a “ruler” line between these two points. The line is labeled (L1, L2, etc.).

c. When you hover over the measurement, the line turns red, the measured length is displayed on the image, and a message appears telling you to “right-click to edit.”

d. Right-click on the line. A Ruler Properties edit box opens. In this box you can: change the name of this measurement, set the tolerances for recycle and reject, change the “accuracy”(number of sampling points along the line).

Ruler Properties

e. You can click on the ruler icon in the Ruler Properties edit box, to set units on the measurement.

f. Click on the “check” button to accept changes and close the Ruler Properties edit box. Click on the red X button to close without changes.

### Using Circle-Line

Circle-Line measures the distance between a circle center point and an edge. You need to have already defined circles using the Circle tool.

Highlighted Edge

1

2

a.	Click on the “Circle-Line” button. Move the mouse over the image in the Work Area. You should see the centers of circles highlighted as you move across them. These are “point candidates” that you can use with Circle-Line. Valid points are Circle centers.

b.	Click on the center point of a circle, and a “rubber” line appears that follows your cursor movement. Click on the edge that matches your measurement, and iIS draws a “ruler”line between these two points. The line is labeled (L1, L2, etc.).

Drag here to

resize the

search area

c. You can drag the endpoint on the edge if it is not exactly where you wanted. The small box that appears around the endpoint defines the search area for locating the edge at that endpoint. You can drag the solid square at one corner of the box, to resize the box and increase the search area; but searching more area increases the processing time.

d. When you hover over the measurement, the line turns red, the measured length is displayed on the image, and a message appears telling you to “right-click to edit.”

e. Right-click on the line. A Ruler Properties edit box opens. In this box you can: change the name of this measurement, set the tolerances for recycle and reject, change the “accuracy”(number of sampling points along the line).

Ruler Properties

f. You can click on the ruler icon in the Ruler Properties edit box, to set units on the measurement.

g. Click on the “check” button to accept changes and close the Ruler Properties edit box. Click on the red X button to close without changes.

### Using Point-Circle

Point-Circle measures the distance between a point and a circle center. You need to have already defined circles using the Circle tool.

a.	Click on the “Point-Circle” button. Move the mouse over the image in the Work Area. You should see the points of angles, line midpoints, or the centers of circles, highlighted as you move across them. These are “point candidates” that you can use with Point-Circle. Valid points can be Circle centers, ruler midpoints, or the points where Angle measures meet.

b.	Find two points that match your measurement. Click on one, and a “rubber” line appears that follows your cursor movement. Click on the other, and iIS draws a “ruler” line between these two points. The line is labeled (L1, L2, etc.).

c.	When you hover over the ruler line, the line turns red, the measured length is displayed, and a message appears telling you to “right-click to edit.”

d.	Right-click on the line. A Ruler Properties edit box opens. In this box you can: change the name of this measurement, set the tolerances for recycle and reject, change the “accuracy”(number of sampling points along the line).

Ruler Properties

e. You can click on the ruler icon in the Ruler Properties edit box, to set units on the measurement.

f. Click on the “check” button to accept changes and close the Ruler Properties edit box. Click on the red X button to close without changes.

### Using Concentric Circles

Concentric Circles measures the distances between the actual centers of two or more circles, and the averaged center point of all circles. This tool reports 0 if all circles are perfectly centered (distance = 0). This tool ignores selecting the same circle more than once for concentric measurement.

a.	Click on the “Concentric Circles” button. Move the mouse over the image in the Work Area. You should see the edges of circles highlighted as you move across them. The Circles must already be defined using the Circle tool, and must have a measurement enabled. The Circle tool defaults to no measurements enabled.

b.	Click on the outer edge of one circle, then click on the outer edge of the other circle, then double-click to stop the drawing. iIS draws a (yellow) circle inside the two existing circles. The circle is labeled (CC1, CC2, etc.).

c.	When you hover over the inside circle, the circle turns red, and a message should appear, telling you to “right-click to edit.”

d.	Right-click on the circle. A Concentricity Properties edit box opens. In this box you can rename the measurement and set the tolerances for recycle and reject.

e. Click on the “check” button to accept changes and close the edit box. Click on the red X button to close without changes.

Concentricity Properties

### Using Count

Count looks for light objects on a dark background, or dark objects on a light background. The background does not have to be uniform. This tool is sensitive to changes in light conditions.

a.	Click on the “Count” button. Move the mouse over the image in the Work Area.

b.	Click and release, move the mouse, and click again, to draw a rectangle over the area where you want to count objects. iIS draws yellow boxes around the objects found. The rectangle is labeled (N1, N2, etc.). You can drag the corners to resize the rectangle.

c.	When you hover over the rectangle, the rectangle turns red, and a message should appear telling you to “right-click to edit.”

d.	Right-click on the rectangle. A Count Properties edit box opens. In this box you can: rename the measurement, set the tolerances for recycle and reject, specify the object size or area, select bright objects on a dark background, or dark objects on a bright background, and change the sensitivity. A higher sensitivity is more sensitive to defects or light changes. A zero in the maximum field for Width, Height or Area means there is no maximum limit. The Area, Length and Width are in calibrated units, or pixels if calibration was not performed.

e.	Click on the “check” button to accept changes and close the edit box. Click on the red X button to close without changes.

Count Properties

NOTE	The Area, Width and Height are initially set to 10% of the objects first detected when you draw the ROI. These may need adjusting, depending on lighting or the range of defects. Objects smaller than the minimum settings, or larger than the maximum settings will be ignored. The size of a reflection can change greatly.

### Using Edge Count

Edge Count counts the number of edges crossed by the ROI (Region-Of-Interest) outline. This tool is sensitive to changes in light conditions.

a.	Click on the “Edge Count” button. Move the mouse over the image in the Work Area.

b.	Select the ROI (Region-Of-Interest) Click and release, move the mouse, and click again to draw the ROI through the area where you want to count the edges. The ROI is labeled (E1, E2, etc.). You can drag and resize the ROI.

c.	When you hover over the edge of the ROI, the ROI turns red, and a message appears telling you to “right-click to edit.”

d.	Right-click on the ROI. An Edge Count Properties edit box opens. In this box you can select the different measurement tabs and enable measurements, rename the measurements, set the tolerances for recycle and reject, change the edge sensitivity. “Dark-Light” means a transition “from-dark-to-light” and “Light-Dark” means a transition “from-light-to-dark”. “Either” means both transitions.

Count	count the number of edges crossed by the outline of the ROI.

Avg. Separation	measure the Average Separation or average distance between edges.

Min	measure the minimum separation or minimum distance between edges.

Max	measure the maximum separation or maximum distance between edges.

e.	Click on the “check” button to accept changes and close the edit box. Click on the red X button to close without changes.

Edge Count Properties

### Using Intensity

Intensity counts the number of pixels at each intensity value and calculates results based on the counts. Intensity detects both light and dark artifacts. This is useful for detecting defects such as discoloration and scratches in a region that is supposed to be uniform in color. This tool is sensitive to changes in light conditions.

a.	Click on the “Intensity” button. Select the ROI (Region-Of-Interest) shape. Move the mouse over the image in the Work Area. There should not be any highlighted edges.

b.	Click and release, move the mouse, and click again, to draw a line, rectangle or circle on the image. The ROI is labeled (M1, M2, etc.). You can drag to resize the ROI.

c.	When you hover of the edge of the ROI, the ROI turns red, and a message appears telling you to “right-click to edit.”

d.	Right-click on the ROI to open a Statistics Properties edit box. There are four tabs for the four measurements. Use the tabs and “Enable” check box to enable the measurements. Statistics Properties

Statistics Properties

Mean	Mean (average) pixel intensity value in the ROI.

Min	Minimum pixel intensity value in the ROI.

Max	Maximum pixel intensity value in the ROI.

Std Dev	Standard deviation of pixels intensity values in the ROI.

e.	Click on the “check” button to accept changes and close the edit box. Click on the red X button to close without changes.

When you are finished defining measurements, Click OK to exit the Tools panel.

In the Setup Measurement panel, click OK. In the Setup panel, Measurement is checked.

## Label

### Set up the Inspection

a.	Click on the “Label” button.

b.	Place a product with a good (or perfect) label in front of the camera, in the location where your inspection parts will appear, or a series of good parts moving past the camera.

c.	Click on “Take a Picture” and look at the still image. You will use this image to set up your measurements and tolerances.

First, snap a “still” picture of the label.

Set up the inspection area and search area for the label.

Set up the barcode reader if you need to read a product barcode.

Set up the inspection methods and tolerances.

Finally, you train on several good examples of your label.

Exit the measurement setup panel and go back up one level.

Label Setup Panel

### Set up the Inspection Area

a.	Click on the “Inspection Area Settings” button.

In the Inspection Area Setup, you tell the iIS what area to inspect, and how much the position of inspected parts will vary. The more parts vary in position, the more time it takes to locate them before making measurements. If possible, minimize part movement with good fixturing.

Define the actual area of the label, using the blue or the Inside box, in the image area.

The first pair of numbers is the top left corner of the active (selected) box in the image.

The second pair of numbers is the width and height of the active (selected) box in the image.

Add or subtract to the area to be ignored.

Exit this panel and go back one level.

Specify if the inspection parts vary in their position, using the red or Outside box, in the image area.

Define any areas to ignore, using the grid boxes. Green area is inspected, pink area is ignored during inspection.

Inspection Area Setup Panel

b.	Click on “Set Inspection Area”. Adjust the size of the (Blue) Box to enclose the area of the label you are inspecting.

c.	Click on “Set Movement Area”. Adjust the size of the (Red) Box in the image area, to enclose the area where the label is allowed to appear. This is the “search area.” The Movement Area must be at least 8 pixels wider and taller on each side of the Inspection Area.

NOTE	A bad (inaccurate) trigger or variable speed line can create a large variation in the label position.

Inspection Areas

d.	Click on “Set Inspection Grid”. A green grid appears over the label, and the “Remove Cells” button is highlighted; to create areas to ignore.

e.	Click on individual rectangles, or drag to select multiple rectangles in a line, or a rectangle. The area of pink rectangles is ignored during learning or training. You want to ignore areas that vary or change on your label, like dates, lot codes, or punch-outs. Use the Add Cells and Delete Cells to add or subtract from the inspected/ignored area. The Don’t Care areas must be inside the Inspection Area (label).

f.	Click OK to go back to the Setup Label panel.

### Set up Barcode Readings

Setting up a barcode reading is optional.

a.	Click on Barcode Setup. Click on the check box beside Enable Barcode Reading.

You can use barcode reading to verify the code is always the same, and report errors only. Or, you can use barcode to read the value of changing codes, and report every read value. You can pre-select the barcode type, or let iIS auto-detect the code type.

b.	Complete the barcode settings and test.

c.	Click OK to return to the Setup Label panel.

Click to enable the barcode reader.

Clear this box if the barcode changes, or if you want to ignore the barcode during the label area inspection.

This box displays the barcode type found if you used Auto Detect.

Select the reporting method you need.

Enter the value expected for all barcodes, if you checked NO.

If you know the type of barcode, use the list box to select the correct type. Use Auto Detect if you know the code but not the type.

Click NO if the barcode is always the same number.

Click YES if the barcode value changes.

Click Read to test your settings. The barcode value read is displayed below the button. Click Clear if the value was wrong, and adjust your settings.

Exit this panel and go back one level.

Barcode Setup Panel

### Set up Inspection Settings

a.	Click on the “Inspection Settings” button.

Define the allowable rotation, allowable intensity changes, and differences between the trained images and the inspection images.

NOTE See the comparison of “Acceptable Variation” and “Acceptable Defects” on the following page.

b.	Complete the Inspection Settings, and click OK.

Define how much the label is allowed to rotate, or how much rotation causes a failure. Click on the solid circle/button, and drag it around the outer circle. You can also use the arrow keys.

Define what percent of the total label area is allowed to change; or how much “defective” area causes a failure.

If the label position does not change you can turn off the Search process (locator). This makes the iIS and Solution run faster.

Define how much the pixel intensity is allowed to change between images. Range is 1 to 255 gray levels (GL). This setting affects learning and inspecting.

Exit this panel and go back one level.

Inspection Settings Panel

Once you have clicked on a slider, you can also use your keyboard’s arrow keys to step the slider up or down.

### Defect Tolerance in Inspection Settings versus Learn Images

Affects Learning only

Affects Inspecting only

Affects both Learning and Inspecting

Affects Inspecting only

Comparison of Inspection and Learn

Percent of Acceptable Label Variation	-	iIS learns the difference between image samples. You decide how much of a difference is acceptable. If you select zero percent, iIS will learn on every new label that is not identical (0% difference) to what has already been learned. A value of 1% means iIS will learn on every new label that has an area of 1% (or more) different from any of the labels already learned.

Percent of Acceptable Label Defects	-	During the inspection process, the tolerance can be set as low as during the learning, or even lower. A setting of 0% means only pass labels that are not different from all of the labels that were learned. A value of 2% means pass labels with defects as large as 2% different from all the labels that were learned.

Acceptable Contrast Tolerance	-	A larger number means allow more variation in light on the labels. It is common to train (Learn) on a value of 16, and inspect on a value of 24. Increasing the value of “Acceptable Contrast Tolerance” will allow more labels to pass during inspection.

### Setup Model Learning

a.	Click on “Learn!”

b.	Select the “Percent of Acceptable Label Variation” and get ready to present or show several examples of good labels, or products with labels, to the iIS.

c.	Click on “Learn Good Parts.” iIS first uses the still image to build a model of how it sees your label. The build process may take a couple of minutes to complete, depending upon the complexity of the label being inspected.

NOTE	During this learning process, the rejection mechanism should be disconnected.

d.	When the status changes to “Built” iIS is ready to learn the range of acceptable labels from multiple samples. Start showing more parts to the camera. If you are triggering externally, such as from a conveyer belt, start the conveyer but disable the rejection mechanism.

This progress bar shows when iIS has built the initial model of your label. The text on the right changes from “not built” to “built”.

Clear the control unit memory of learned images and variations.

Train on the still image, then on a series of good labels. You must present several labels so iIS can learn the range of acceptable variations.

This window shows the progress of learning from multiple labels. The black wavy line should start out above the red solid line, then settle out to below the red line.

Exit this panel and go back one level.

Add the sample to the learned images if the difference is above this value.

Learn Images Panel

### Readjust the Acceptable Contrast Tolerance

a.	Click on Inspection Settings.

b.	Now that iIS has learned about your labels, you can readjust the Acceptable Contrast Tolerance to define the acceptable variation in light during an inspection run.

c.	Click OK to return to the Setup Label panel.

d.	In the Setup Label panel, click on OK to return to the Setup panel.

## Setup Decision Logic

a.	Click on Decision.

b.	Define what action to take in the event that parts are skipped (as a result of insufficient time allocated to the processor. See the discussion on “Things to know about Processing Time”).

c.	Gauge Application: Define the conditions for part acceptance, recycling or rejections. The “decision table”shows all the measurements you defined in the “Setup Tools” panel. NOTE: If a measurement is not shown, it may not be enabled in the Properties Edit box.

Exit this panel and go back one level.

What to do when parts fail excessively: Continuous means one after the other, successive failures “in-a-row”.

What to do with images that are “skipped”“dropped” or “missed”. If images or labels are coming faster than it takes to Search and Inspect, some images will be missed.

You can change all the settings and criteria. You can choose operators AND, OR, IGNORE from pull-down lists in each box in this table.

GPO0, GPO2, GPO3 are the signals output by the control unit.

Each measurement name you defined appears in the left column.

The default decision setup is:

Pass only if all measurements pass.

Recycle if any one measurement is in the Recycle category.

Reject if any one measurement is in the Fail category.

Gauge Application

Decision Panel

d.	Click OK to return to the Setup panel.

## Set up Outputs

a.	Click Outputs.

In this panel, you can Enable or Disable saving images and data in the control unit memory (History Log). You can set the pulse duration of the general-purpose outputs (used for Pass, Recycle, and Reject signals), and you can set a delay between the end of processing and the output of these general-purpose pulses. You can also enable “Conditional Outputs” or “Reporting” data strings to the Serial Port or Ethernet Port.

Enable or disable saving history logs to the control unit memory. Eight records are saved in each category: Pass, Fail, Recycle.

Enable Conditional Outputs to define and enable output to the Serial Port or Ethernet. The Setup button is not active if Disable is selected.

Define the duration (pulse width) of the four control output signals (General Purpose Outputs). You must click Apply to accept your change.

Exit this panel and go back one level.

Define if the output signals occur immediately, or after signal from a sensor. Use the Rejector Setup button to adjust the delay. The button is not active if the decision output is immediate.

Outputs Panel

Once you have clicked on a slider, you can also use your keyboard’s arrow keys to step the slider up or down.

## Set Up Rejector Timing

If you are using the iIS to control the final movement of parts on your line, click on “Rejector Setup”. If it is not active, click beside Decision Sensor, then Click Rejector Setup. Rejector Timing does not operate if you are in Internal Timing mode. You must be using an Inspection Trigger and a Decision Sensor to use the Rejector timing.

The control unit stops processing whenever you enter this panel, and begins rejecting parts according to the number entered in the “Reject Rate” box. The default is 0, to reject no parts (pass all parts). You can enter a number and click Apply. Enter 1 to reject every part, enter 2 to reject every other part, etc. Use the slider to adjust the delay between the part in place (for rejection) sensor and the actuation of the rejector pulse.

Enter a number, then click Apply to begin rejecting parts. Adjust the slider so the rejector activates at the correct time.

Use the slider to adjust the amount of delay after a signal from a sensor. The sensor would indicate

the part is in place to be rejected.

Exit this panel and resume previous run state.

Rejector Setup Panel

If you have a “kicker” you want to make sure the kicker hits the part in the middle, and cleanly moves it off the line. If you have a flipper or track changer, you want a change at the last instant just before the product moves onto the new track, and not hit or disrupt the part. If you have a marker, you want to get your mark in the middle of the label, not partially off the edge of the label. Please refer to the following discussion and instructions.

## Rejector Delay

The Rejector is a delay mechanism controlling the Pass, Recycle and Fail signal pulses output on GPO0, GPO2 and GPO3. The intended purpose of the rejector is to remove defective parts from the production line at the correct point in time. The rejector can operate in two modes: “Immediate” and “Decision Sensor”. Immediate mode is intended for configurations where the outputs will be the input to another device such as a PLC (Programmable Logic Controller) that will schedule the rejection of the part. If no PLC is to be used, Decision Sensor mode allows for scheduling the rejector output to remove the defective part as shown in the following illustration.

Rejector Function

The illustration shows a typical rejector setup using the decision sensor. If immediate mode had been selected, input from the Rejector Trigger would be ignored and the outputs would activate immediately upon a decision being available. The Rejection Delay is the time it takes for the part to move from the decision sensor (Part-in-Place Sensor) to the device that will kick the part off the line. This time interval must be predictable and constant. The time value is input to iIS from the Rejector Setup panel, accessed from the Setup Outputs panel.

## Rejector Setup Procedure

a.	Go to the “Setup Outputs” panel.

b.	Select the “Decision Sensor” radio button in the “Decision Output” section.

c.	Click the “Rejector Setup” button. This will open the Rejector Setup panel. Here you can adjust and test the rejection delay. The rejection delay is the time from the decision trigger to when the part arrives at the kicker or rejector mechanism. This is shown in the previous illustration.

d.	Set the Reject Rate for your rejector test. For example if you set this to 1 every part passing the rejector trigger will result in the GPO3 (Reject) outputing a pulse. Every part will be rejected. Entering 2 will reject every other part. Adjust the Rejection Delay to a value that properly rejects the parts at the rejection frequency selected.

Caution:	When adjusting the Rejection Delay, allow the rejector to resynchronize with the parts, by allowing at least 1 part to pass between the rejector sensor and the kicking device; this is the current (old) rejector delay. Also be aware that if parts are equally spaced, it is easy to make the error of adjusting the rejector so that it is off by 1 or more parts; rejecting the wrong part.

e.	When you select “OK” from the rejector setup panel, the line must be reset. To do this, you must know how many parts are between the Inspection Trigger part-in-place sensor and the Rejector Trigger part-in-place sensor (parts 5 through 9 in the illustration). These parts will be “rejected” according to the Reject Rate set in the Rejector Setup panel.

Caution: If this value is not correct you will reject the wrong parts. The safest method (if pos-sible) is to stop the transport and remove all parts between the two sensors, and leave the part count in the dialog box at “0”.

f.	Click “Restart”.

g.	Reconnect the Inspection Trigger input, and restart the product transport. The “testing” and rejection stops, and the iIS goes back to processing (if it was running) or paused for programming.

NOTE	When you click the Reset Product Line button on the Monitor/Run panel, you again tell the iIS how many parts there are between the Inspection Trigger and the Rejection Trigger. These parts are “Passed”.

## Set Up Reporting

If you want to output results of each inspection to another device, for control, logging or analysis, you will need to set up the reporting method.

a.	Click the Enable radio button under Conditional Outputs, and then click “Setup”.

Here you select or define strings that are output by the iIS control unit, conditions that cause strings to be output (or which condition causes which string to be output), and where (which port) the strings are output. When the iIS is running and inspecting, if your condition is True, a string is sent (output). If you always want data output, enter a number (2, 3) in the “If Condition” field. 1 already exists.

Click Add and Delete to change the list of Conditions, strings, and destinations in use.

Select a pre-defined destination, or click Add/Edit to edit a destination or define your own destination.

Exit this panel and go back one level.

The strings are referenced by names. Select a pre-defined string, or click Add/Edit to edit a string or define your own string.

Select a pre-defined Condition, or click Create/Edit to edit a Condition or define your own Condition statement. Enter a number (2, 3) if you always want output.

If your Condition is True, a string is output.

Port 5020: for your use, your formating. Ports 5021 and 5022 are reserved for the History Log output.

Conditional Output Panel

NOTE	To view data, you can use the History Log page or set up your own HyperTerminal session. These are described later in this manual.

NOTE	Port 5020 is for you to use, to define your own data format. Port 5021 and Port 5022 are reserved for use with the History Log output.

### Create Condition

b.	In the Conditional Output panel, click “Create/Edit” to edit the current condition, or create a new condition statement.

You can type text in the Condition field, insert variables, constants and operators.

The Result variable takes the integer values: 0=Ignore, 1=Pass, 2=Recycle, 3=Reject. Result makes a good condition. The defined measurements take floating-point numbers. These do not make good Conditions.

c.	Click OK to return to the Conditional Output panel.

Click on a variable name, then click on Insert Variable to add it to the Condition statement. Type in a Constant (number) and click Insert to add it to the Condition statement. Click on the boolean and arithmetic operators to add them to your Condition statement.

Your Condition Statement appears here. You can also type in this field. Click on Clear to empty the Condition statement field.

0=False, 1=True

Click on Evaluate Condition to see the current value of your condition to the right of this button.

Exit this panel and go back one level.

Create/Edit Condition Panel

### Select or Define New Strings

d.	Click the Add/Edit button that appears beside “Then Send” to define new strings in the Create Strings panel. Or use the list box to select a pre-defined string.

Here you can select pre-defined strings or define your own strings. Strings are called by their String Names. Enter a new name, then use the Variables and Special Characters to create your string. Strings contain values separated by commas or spaces, and ending with special characters. The Format field defines how variables will appear in your output string.

e.	Click OK to return to the Conditional Output panel.

Click Add to create a new string, or use the list box to select an existing string.

Click on a variable name, then click on Format to change how the output value appears, then click Insert Selected Variable to add the variable to your string. Click on a special character and click Insert to add it to your string.

The defined content of a string appears here. You can type commas and spaces in this field, to separate variables. The value of the variables will be substituted in the output string.

Exit this panel and go back one level.

The evaluated string value appears here. The number or boolean values are substituted for the variables you inserted.

Add/Edit Output Strings Panel

### Select or Define New Destinations

f.	Click the Add/Edit button that appears beside “To Destination” to define new destinations in the Create Destinations panel, or use the list box to select a pre-defined destination.

You can select a pre-defined destination, or define your own destination.

 COM3 is the serial output port on the RS-232C connector.

 TCP/IP is for the Ethernet Port. You define the role of the iIS control unit as a Client or Server.

Client means the control unit sends data to a network server. You must enter the IP Address of your server. Server means the iIS control unit is a server, and computers request data from the control unit. Use the port number 5020 for those requests. Requests for data do not show any impact on the inspection speed. Ports 5021 and 5022 are reserved.

g.	Click OK to return to the Conditional Outputs panel.

h.	Click OK to return to the Output panel. Click OK again to return to the Setup panel.

Set the port settings for communications on the serial port. Click Apply or Restore Defaults. COM3 is the only valid serial port.

Define output on the Ethernet connection. If you choose Client, enter an IP Address destination. If you choose Server, your requesting devices should use the port number shown (5020). Click Add to add your new destination to the list.

Exit this panel and go back one level.

You can delete a destination by clicking on the destination and then clicking Delete.

Add/Edit Destination

## Ready to Start Processing

a.	Click on “Ready to Run Hit to Start” to start inspecting parts. The iIS begins inspecting parts immediately and the button text changes. Also notice that the top of the panel turns Green, meaning the iIS is processing or inspecting. Any other color means the iIS is in a stop or pause state.

b.	Click OK to return to the Home panel.

The text on this button changes with the state of the iIS and Solution.

Ready to Run (Gauge / Label)

## Monitor Inspection Results

a.	Click on the Monitor button to see the results of inspecting.

The counters at the top of the Run panel should be changing. The Inspect Time is displayed in the upper right corner of the panel. You can reset the counts at any time. You may not see the counts go to zero, but you will see them go to a smaller number. There is a small delay between clicking on the Reset Statistics button and the change in numbers.

Updating the Client display with images affects the total processing time. The iIS may run much faster if you disable the Image update by selecting “Data Only” or “Nothing” under Display in this panel.

Current measured values.

Stop the processing and output signals, and view History records for inspected labels. This button is not active if History Log was Disabled in the Setup Outputs panel.

Exit this panel and go back one level.

Define the contents of the Client Display: Image and Data, Data only, no display update. Selecting Data Only may reduce the total processing time.

This “Status bar” changes to Green

Total Processing Time = Inspect Time + 40 ms.

If you are using a rejector sensor, this button is active, and allows you to pause the iIS and tell it how many parts are on the line.

An Exclaimation Point may appear if there is a problem, for example with the rejector sensor. Click on this to display a message.

Reset the counters on this panel.

Save an image to the directory previously defined.

Run Panel (Gauge)

NOTE	When you click the Reset Product Line button on the Monitor/Run panel, you must tell the iIS how many parts there are between the Inspection Trigger and the Rejection Trigger. These parts are “Passed”.

Label Application: In the Image Display area, the red box shows the search area, and a blue cross hair shows the center of position of the original trained model image. The Green cross hair shows the displaced center of the current label, and the blue box is the inspect area. The two green boxes represent unique areas identified by iIS in building its model. These are used to find the displacement and angle. Updating the Client display with images affects the total processing time. The iIS may run much faster if you disable the Image update by selecting “Data Only” or “Nothing”under Display in this panel.

Stop the processing and output signals, and view History records for inspected labels. This button is not active if History Log was Disabled in the Setup Outputs panel.

If you are using a rejector sensor, this button is active, and allows you to pause the iIS and tell it how many parts are on the line.

Exit this panel and go back one level.

This “Status bar” changes to Green

Total Processing Time = Inspect Time + 40 ms.

Define the contents of the Client Display: Image and Data, Data only, no display update. Changing to Data Only may reduce the total processing time.

An Exclaimation Point may appear if there is a problem, for example with the rejector sensor. Click on this to display a message.

Reset the counters on this panel.

How much of the Label’s area is passing (%).

Monitor Panel (Label)

### Review History

a.	Click on the “History Recall” button if it is active. This button is not active if the History Log was disabled in the Setup Outputs panel. We recommend disabling the history log during actual inspection.

The History panel displays the measured values or data. The Work Area displays the image for each part that was recorded. The maximum number of records is 8. The last 8 records are saved for each category. Use the drop down to view Pass, Recycle, or Reject records.

b.	Click on OK to return to the Run panel. Note the Inspect Time.

c.	Click on OK to return to the first panel.

Select a category: Pass, Recycle, Reject.

Exit this panel and go back one level.

The values for each measurement are displayed.

Select a Record Number to view. The last 8 records are stored in control unit memory.

History Recall Panel (Gauge)

NOTE	History records are not cleared when you select Reset Statistics in the Monitor/Run panel. It is possible to view outdated records. To clear the history records:
1) save your Solution (if you have not already done so),
2) go to Select Solution and click on Start a New Solution,
3) go to Select Solution again, and click on Load Last Saved Solution.

Click on OK to return to the Run panel. Note the processing time. Click OK again to return to the first panel.

## Save and/or Export your Solution

a.	Click on the “Save/Export Solution” button.

b.	Enter a short description.

c.	Click on the “Save Solution” button.

You should save your Solution to the control unit memory, before you try optimizing the inspection time settings. The last running Solution (usually ID = 00, or the Solution that matches the hardware switch ID selection, if present) automatically runs each time your control unit is started or booted. You can Export (save) your Solution to your PC or to a network device. Exporting a Solution allows you to distribute it across multiple control unit.

Save your solution to control unit memory.

Select the current running solution, any one Solution, or all Solutions saved on the control unit, to Export.

Exit this panel and go back one level.

Enter a Description that you will recognize.

Progress bars

Assign an ID number to the Solution (00 to 15) Up to 16 Solutions can be saved on the control unit.

Select the current running solution, any one Solution,or all Solutions saved on the control unit, to delete.

Save your solution to your PC or a network device.You must save your solution to the control unit memory first, before you Export.

Save or Export Solution Panel

NOTE	To run Solutions 01 to 15, you must supply a 4-bit ID code and a “Change Strobe” pulse on the I/O Connector. If you do not have a way to supply the 4-bit code, save and then Export each Solution with ID = 00.

You can import any Solution with ID = 00 to run it. You may now want to return to the Setup Sensor panel to adjust the Internal Timer or cycle time. If the time is set too low, images or parts may be skipped (skipped parts follow the action defined in the decision panel).

## Resetting the control unit

In the extreme case, iIS can get "stuck" constantly processing (LED2 constantly on) and you may encounter trouble accessing the control unit. If access problems persist, you can press the Reset button on the control unit using a pencil, pen, or small screwdriver. Close your Internet Explorer. The control unit will reboot in a minute, and begin running the Solution. Open Internet Explorer, and open the iIS Home Page. Click on "gauge" or "Label", and to the Sensor Setup panel and increase the time. Save the Solution to memory and reboot. You should close your Internet Explorer when you reboot the control unit. Open your Task Manager and make sure there is no iWorks.exe process running after you close Internet Explorer. The control unite takes approximately one minute to reboot. Look for the Server Heartbeat LED (LED1) to start flashing.

## Close the Application Window

If you are satisfied with your Solution and have saved it, you can return to the first panel and Click on Close to exit the iIS application window, and return to Internet Explorer. The iIS continues running and inspecting parts.

NOTE	You cannot restart the client application from the Gauge/Label page in Internet Explorer. You must refresh or move to another page first. To reopen the Client window, click on Home, then click on Gauge/Label in the home page.

## Monitoring Inspections from the Web

a.	Click on Monitor. The Monitor screen opens within Internet Explorer. Only one person can access the iIS at a time, whether the session is Monitor or full access. The Monitor Window is not password protected. History viewing or production line control are not available from the monitor window.

b.	When finished, scroll down Internet Explorer and click on the Home button. This closes the Monitor window. Close Internet Explorer to ensure access to the iIS is released.

Monitor Window (Gauge)

Monitor Window (Label)

## Viewing Data Output

If you enabled Conditional Output, there are several ways to view data output. This section will describe using the History Log page, and using HyperTerminal.

### History Log

If you enabled conditional output, and the control unit is a Server, you can use the History Log link on the iIS Home page.

a. 	On the iIS home page, click on “History Log”.

b.	On the History Log page, click the “Connect” button, then click the “Start Logging” button.

c.	Before you close or leave this page, click the “Stop Logging” button, then click the “Disconnect” button.

History Log Page

### HyperTerminal

If you enabled conditional output, and the control unit is a Server, you can use HyperTerminal. You can use HyperTerminal for either Ethernet or Serial communication.

a.	Open a new HyperTerminal session, enter a name for the session, for example, “iIS1”,  select icon, and click OK.

b.	In “Connection Description” enter your area code.

c.	For Ethernet: Enter the IP Address of the control unit in the “Host Address” and enter the port number (iIS control unit default is 5020). Select “TCP/IP(Winsock)” for the “Connect using”. For Serial Port: Select COM1 or COM2 (your PC serial port) for the “Connect using”. Change the Port Settings to match the values you set in the Add/Edit Destinations panel.

d.	Click OK, and the data should immediately start scrolling in the HyperTerminal window. If not, double-check your port settings in HyperTerminal. If there is still no data, verify that a Solution is active and running, and conditional output is enabled in the Setup Outputs panel.

Head Office and Factory

4-7-1, Asahigaoka, Hino-shi, Tokyo, 191-0065, Japan

Phone: +81-(0)42-589-8771, FAX: +81-(0)42-589-8774

Kansai Branch

Nitto Building, 2-1-2, Sakaemachi-dori, Chuo-ku, Kobe, Hyogo 650-0023, Japan

Phone: +81-(0)78-321-3461, FAX: +81-(0)78-321-3463

Fukuoka Office

Yen Building, 3-7-21, Hirao, Chuo-ku, Fukuoka 810-0014, Japan

Phone: +81-(0)92-523-3395, FAX: +81-(0)92-523-3397

iIS-010 series - Operation Manual (Application)           June 29 2004 (Draft)